3 October 2015

Isle of Wight catchment

River Basin Management Plan

Cycle 2, 2015-21

- What Isle of Wight waterbodies are failing for according to the Water Framework Directive

- Objectives for cycle 2 of RBMP (2015-21)

- Likely action to address the fail

WFD classifications for surface water, are a pass: High or Good, or three levels of fail: Moderate; Poor;  Bad. Groundwater is rated as Good or Poor.

Objectives are to address all the issues listed for each waterbody by the date specified. The target is to get to Good Ecological Status GES (if the waterbody is not modified) or Good Ecological Potential GEP (if modified).

Lukely Brook GB107101006250

Objective is Good by 2027.

Overall status in cycle 2 is Moderate.

| Issues | Actions needed |
| --- | --- |
| Fish are Moderate. The main reason is the physical modification of the river which has resulted in poor habitat quality and lack of connectivity. Fish monitoring will continue in cycle 2. | St Cross Mill and Westminster Mill and West Mill (or Carisbrook Gauging Station F/P) have been included on the “Solent Fish Pass Programme” – which has no funding in 15/16. Southern Water is investing in 5 structures and impediment to flow is expected to be announced in 15/16. |
| Mitigation Measures for channel modifications are Moderate. It is heavily modified and has many obstructions to fish migration | Five schemes have been proposed for inclusion in Southern Waters AMP6 NEP to help address some of these issues. [Southern Waters investigation identified that their abstraction does reduce flow, but there is no evidence of any lasting damage on the ecology]. At Dairycrest site by Sainsbury’s, the Barratts housing development includes bank improvements on a 120m section. Approx ¾ of concrete bank being removed, with the river being widened from about 4 to 7m. |
| Hydrology is Moderate. Southern Water investigations identified that their abstraction does reduce flow, but there is no evidence of any lasting damage on ecology. | See action above. |
| Macrophytes and Phytobenthos Combined are Moderate. | Suitability of sampling on this waterbody to be investigated, results to be know in 2015 (EA A&R team) |
| Non native invasive species, Japanese knotweed and Himalayan Balsam, been tackled as part of Plant Positive. | Removal needs to continue, based on the Defra funded LAG removal. Funding? |