3 October 2015

Isle of Wight catchment

River Basin Management Plan

Cycle 2, 2015-21

- What Isle of Wight waterbodies are failing for according to the Water Framework Directive

- Objectives for cycle 2 of RBMP (2015-21)

- Likely action to address the fail

WFD classifications for surface water, are a pass: High or Good, or three levels of fail: Moderate; Poor;  Bad. Groundwater is rated as Good or Poor.

Objectives are to address all the issues listed for each waterbody by the date specified. The target is to get to Good Ecological Status GES (if the waterbody is not modified) or Good Ecological Potential GEP (if modified).

River Medina GB107101005990

Objective is Moderate by 2021, Good by 2027, because of naturally occuring iron.

Overall status in cycle 2 is Moderate. It is a heavily modified waterbody (HMWB).

| Issues | Actions needed |
| --- | --- |
| Invertebrates are Moderate. Reason are - the poor natural background conditions (naturally occurring iron), also potential for sediment to get into waterbody via land drainage channels. There have been no detected STW impacts on water quality. - Monitoring site has been moved from fast-flowing Shide weir, which is straightened, to downstream of Cridmore bog at Appleford Farm | If actions below are addressed, waterbody may get to Good despite naturally occurring iron: - Action to undertake agricultural visits to identify the issues and develop solutions with the landowners. - EA analysis team to check new chemical monitoring for issues/ failures. |
| Macrophytes and Phytobenthos sampling to be considered. | Investigation underway (no result yet, Oct 15) |
| Fish monitoring will continue in cycle 2. |  |
| Mitigation measures for channel modifications are Moderate. | - The River Medina Fish Pass Project, structures identified that are as causing an impediment to fish passage. - Actions for habitat improvement identified in the "River Medina Habitats assessment" upstream of Blackwater put forward for implementation. - Potential for softer engineering has been identified at two locations in Newport. - Possibility of getting Defra funding for urban diffuse pollution. |
| Non native invasive species – part of the Mitigation measures failure. | - Continued management, pulling/ spraying of non-invasive species. - Plant Positive funding is coming to end. New money or initiative is required to continue action on a number of waterbodies, or all the good work that has already been done will be undermined. This funding may come as a measure to reduce flood risk on EA land upstream. |